| Topic | Conference contributions | New contributions for journal extension |
| --- | --- | --- |
| RTL design of MXU | - Hierarchical RTL for matrix multiply unit with PE-core and clocked shell - Each layer in the hierarchy is testbench-verified - Fixed to 8-bit weights and activations (W8A8) | - Parameterizable weight and activation bit-widths - Enables scaling studies and benchmarking of SOTA placement-and-routing algorithms where 8-bit is intractable |
| RTL-to-netlist | - Yosys for RTL-to-AIG (And-Inverter Graph) conversion using default ALU mapping (which uses CMOS-aligned ALUs) - ABC for AIG optimization | - Implemented SiDB-aligned ALUs for adder and multipliers as gate-level netlist, and provided to Yosys for ALU mapping |
| Netlist-to-atoms | - FoM-aware technology mapping to give higher preference to robust & reliable SiDB gates at the cost of higher resulting area - Hexagonalization I/O extension | - Implemented new cost objectives in the gold P&R algorithm which optimizes for SiDB-specific layout rules (which differ from existing cost objectives optimized for other FCN technologies) |
| Experiment | - Compared synthesized MXU against manually estimated blueprints from past studies - Compared uniform vs. FoM-aware synthesis with the W8A8 config, both synthesized using the ortho P&R algorithm | - Compared synthesis results across W8A8, W4A4, and W2A2 - Compared all bit-width configurations with FoM-aware technology mapping - Added gold P&R results for W4A4 and W2A2 thanks to those bit-widths being tractable for gold - Compared new cost objectives in gold to show SiDB-specific improvements |